Geostatistical Visualization of Mineral Distribution

Overview

This project demonstrates the application of geostatistics to model the spatial distribution of mineral concentrations in the Azov region. The workflow includes:

Data preparation from geological research;

Variogram modeling to understand spatial dependence;

Kriging interpolation for mineral prediction;

Incorporation of anisotropy and fault zones to reflect real geological conditions;

The dataset used here is based on real geochemical data from the Azov region, which includes concentrations of Nickel (Ni), Chromium (Cr), Cobalt (Co), Copper (Cu), and Vanadium (V) in various rock types.

Concepts Covered

This project focuses on several key concepts:

Geostatistics: The study of spatially correlated data and modeling techniques like Kriging.

Fault-Controlled Mineralization: Incorporating faults as structural factors influencing mineral concentration.

Anisotropy: The directional dependency of mineral distribution, often controlled by geological structures.

Kriging Interpolation: A method used to predict unknown values based on spatial correlation.

Objective

The main goal of the project is to predict mineral concentration across the study area and visualize uncertainty. Key tasks include:

Data Generation: Using real geochemical data for geostatistical modeling.

Fault and Anisotropy Modeling: Simulating fault-controlled mineralization and directional dependence.

Kriging: Predicting mineral concentrations and visualizing uncertainty.

Methodology

Step 1: Load and Prepare Data

The data, including coordinates and concentrations of various elements, was loaded into a pandas DataFrame. Coordinates were simulated for geostatistical analysis, and Nickel (Ni) concentrations were selected for modeling.

# Load Data

data = load_dataset("data/mineral_data.csv")

Step 2: Apply Fault Effect

Mineral concentrations near fault zones were modified using an exponential decay model to simulate enrichment along the fault line. The apply_fault_effect function was used to simulate the geological process of higher mineral concentrations closer to faults.

# Apply Fault Effect

data = apply_fault_effect(data, fault_x=50)

Step 3: Apply Anisotropy

Anisotropy was applied to the data by rotating the coordinates and stretching them along one direction (controlled by fault orientation). The function apply_anisotropy transformed the coordinates to simulate the fact that mineralization often spreads along fault lines, rather than isotropically in all directions.

# Apply Anisotropy

x_aniso, y_aniso = apply_anisotropy(x, y, angle_deg=30, ratio=2)

Step 4: Compute Variogram

The experimental variogram was calculated using pairwise distances between the sample points. This measured how mineral concentrations varied with distance. We then fitted a spherical model to the data.

# Compute Experimental Variogram

bin_centers, bin_means = compute_variogram(x_aniso, y_aniso, z)

Step 5: Kriging Interpolation

Using the variogram model, Kriging interpolation was performed to predict mineral concentrations at unsampled locations. The predictions were then visualized in a 2D map, and the uncertainty was visualized with a variance map.

# Perform Kriging

grid_x, grid_y, z_pred, z_var = run_kriging(x_aniso, y_aniso, z)

# Plot the Results

plot_kriging(grid_x, grid_y, z_pred, x, y, z)

plot_variance(grid_x, grid_y, z_var)

Faults and Anisotropy Explained

1. Fault Zones:

In this project, fault zones represent structural controls that influence the mineralization process. The effect of the fault was modeled using an exponential function where mineral concentrations increase as proximity to the fault decreases. This is a common phenomenon in geological systems where minerals accumulate around fault zones.

2. Anisotropy:

Anisotropy reflects the directional variation in geological properties. In this case, it simulates the fact that mineralization often follows certain directional patterns, such as fault lines or regional tectonic structures. By applying anisotropy, the project mimics how mineral concentrations are more continuous in certain directions, rather than uniform in all directions.

3. Kriging with Anisotropy and Fault Effect:

By combining both the fault effect and anisotropic properties into the Kriging model, the resulting prediction is more realistic. The mineral concentration is higher near faults, and the interpolation takes into account the directional spread of mineralization, giving a more accurate prediction of the spatial distribution.

Project Structure

geostat-mineral-visualization/

│

├── data/               # Data folder

│   └── mineral_data.csv

│

├── src/                # Source code folder

│   ├── load_data.py    # Data loading functions

│   ├── visualization.py # Plotting functions

│   ├── variogram.py    # Variogram computation

│   ├── kriging_model.py # Kriging model implementation

│   └── geology.py      # Geological modeling (faults + anisotropy)

│

├── main.py             # Main script to run the project

├── requirements.txt    # Dependencies

└── README.docx           # Project documentation

Technologies Used

Python: Programming language

NumPy: For numerical computations

Pandas: For data manipulation

Matplotlib: For data visualization

SciPy: For spatial analysis (distance and variogram)

PyKrige: For Kriging interpolation

Installation

- Clone the repository:

- git clone https://github.com/your-username/geostat-mineral-visualization.git

- cd geostat-mineral-visualization

- Install dependencies:

- pip install -r requirements.txt

- Run the project:

- python main.py

- Future Improvements

- ·  3D Modeling: Expand to 3D Kriging for more realistic geological analysis.

- ·  Multi-Element Modeling: Integrate multiple elements (Cu, Ni, Cr) for simultaneous prediction.

- ·  Interactive Visualizations: Implement interactive maps using Plotly.

- ·  Real Data Integration: Replace synthetic data with actual geological survey data.

- Author

- Vitaly Andriyko
	(Geology + Mathematics + Programming)

- License

- This project is open-source and available under the MIT License.

- This README explains the project’s key steps, from data preparation to Kriging interpolation, while also highlighting the addition of fault zones and anisotropy for a more realistic model. It’s ready to be shared as a professional research-grade project!